Saesoft (R) SIO, Serial Hard/SoftWare IO Convertor - Version 1.1

PII8023 - Free for non-commercial use and distribution.

Usage: SIO [COM-IN{a}] [IRQ-IN{b}] [COM-OUT{x}] [IRQ-OUT{y}]

Example: SIO 4 3 4 5

This Program Version is User Domain Software,

Program(s) is(are) Property of Saesoft International,

(C) Saesoft International 1983 - 1990.	All rights reserved.

SIO, Version 1.1, 25 Aug 1990  01:10:00

This  version  of  the	documentation,	software  and  copyright

supersedes all previous versions of SIO.COM  (COMIRQIO.ZIP)

TRADEMARK

DSZ, Zmodem, PROCOMM+ and PCPLUS+ are registered trademarks.

DISCLAIMER

Use of this program  acknowledges this	disclaimer of  warranty:

"This program  is  supplied as is. Saesoft disclaims all warran-

ties, express  or  implied,  including,  without limitation, the

warranties of merchantability and of fitness of this program for

any purpose. Saesoft assumes no liability for damages  direct or

consequential, which may result  from the use of or inability to

use this program. Even if Saesoft has been advised of the possi-

bility of such damages or any claim by any other party."

SITE LICENSES AND QUANTITY PURCHASES

You do not need a site license for  SIO.COM , this  utility like

many others are "User Domain Software".

+-------+-------+-------+-------+-------+-------+-------+------+

SIO  will convert  any software and hardware interrupt to proper

values in use by communication programs and extended protocol's.

+-------+-------+-------+-------+-------+-------+-------+------+

The best way  to show you  what SIO  can do  for you  is with an

example;

& - SIO  a Serial Hard/SoftWare IO Convertor - Version 1.1  (C) Saesoft  1990 -

Suppose  you have COM 1 and 2  occupied, but  you just	bought a

modem  (in or external), because of  hardware IRQ conflic's, you

can not install your modem on COM 3 or 4... this leaves you with

a problem, COM 1,3,5,7...etc. uses IRQ 4, and COM 2,4,6,8...etc.

uses IRQ 3, the only way to solve this problem is to re-wire the

IRQ used on your modem to a spare IRQ, IRQ 2 or 9 or 5 for exam-

ple, however  this is the easy part,  the next problem, which is

much harder to solve, is to get your software program to use the

new IRQ you just re-wired...

Here is where SIO.COM comes in, after re-wireing your  modem IRQ

to IRQ 5  (in this example),  your communications  program still

assumes that IRQ 3 belongs to COM 4.

Before running any communications program, run SIO.COM	with the

next values: SIO 4 3 4 5

Then run the communications program,  SIO will re-route IRQ 3 to

IRQ 5 and maintain COM 4, the syntax of SIO is;

Usage: SIO [COM-IN{a}] [IRQ-IN{b}] [COM-OUT{x}] [IRQ-OUT{y}]

SIO	   4	       3	    4		 5

And  the communications program  will function	like it should !

further more, any external  protocol  like Zmodem will run with-

out a problem, even in a shell !

+-------+-------+-------+-------+-------+-------+-------+------+

Things to watch for:

There are  many reasons  why SIO doesn't work in some way's, one

of them is the base address  used in combination with a communi-

cations program, if the program depends  on a fixed base address

then the COM port  can not be re-routed, or  if the base address

is in use  by an other	serial card or modem, again the COM port

can not be re-routed.

Communication programs	who do not need  a base address,  can be

re-routed with any COM and IRQ value, this mearly depends on the

type of program your running.

The IRQ  can always be re-routed  without a problem, and this is

the common problem  with most  communication programs,	they are

able to use COM 1 to 4, but only IRQ 3 and 4 !

And  if you have  a program  like PcPlus+  (Procomm+),	then you

still got  a problem,  because PcPlus+	can be configured to any

COM upto 8  and any IRQ upto 7,  but an external  protocol  like

Zmodem will NOT work... (Zmodem 4-11-90 version)

+-------+-------+-------+-------+-------+-------+-------+------+

The next setup is what we use  to get all communication programs

to work on the same COM and IRQ:

PcPlus+ set to COM 4 and IRQ 3,

Zmodem set to "dsz port 4 rz" (is set to COM 4 and IRQ 3),

Our modem is set  to COM 4 and IRQ 5, and to start-up communica-

tion we use the next batchfile:

& - SIO  a Serial Hard/SoftWare IO Convertor - Version 1.1  (C) Saesoft  1990 -

@echo off

sio 4 3 4 5			:set IRQ 3 to IRQ 5 with SIO

call pcplus			:call the PcPlus+ program

sio /u				:unload SIO when exiting PcPlus+

And Zmodem is called inside the PcPlus+ program.

+-------+-------+-------+-------+-------+-------+-------+------+

Note that the COM port has not been changed.

Read "Things to watch for:" for an explanation about this.

+-------+-------+-------+-------+-------+-------+-------+------+

Next is a list of IRQ's and what they are used for:

IRQ  Interrupt	PC/XT  AT  Usage

================================================================

1     0x09	  Y    Y   Keyboard

2     0x0A	  Y    Y   AT slave, could be ro-routed to IRQ 9

3     0x0B	  Y    Y   COM 2,4,6,8,10...etc

4     0x0C	  Y    Y   COM 1,3,5,7,9...etc

5     0x0D	  N    Y   PC=fixed disk adapter, AT=LPT 2

6     0x0E	  Y    Y   Floppy disk adapter

7     0x0F	  Y    Y   LPT 1

8     0x70	  N    N   Real Time Clock

9     0x71	  Y    Y   PC=ok, AT=could be ro-routed to IRQ 2

10    0x72	  Y    Y   PC=unknown, AT=ok

11    0x73	  Y    Y   PC=unknown, AT=ok

12    0x74	  Y    Y   PC=unknown, AT=ok

13    0x75	  Y    N   PC=unknown, AT=80x87 coprocessor

14    0x76	  Y    N   PC=unknown, AT=fixed disk controller

15    0x77	  Y    Y   PC=unknown, AT=ok

This list shows  that on a PC/XT, IRQ 2 or 9 can be used, and if

you are not using LPT 1, IRQ 7 also.

IRQ 10,11,12,13,14 and 15  could work, but  may not be available

on the serial or modem card, but IRQ 2 is quite save.

On an AT/286  or 386, IRQ 2/9 or 5  can be used, and  if you are

not using LPT 1, IRQ 7 also.

IRQ 10,11,12 and 15 could work, but  may not be available on the

serial or modem card, but IRQ 5 and 9 are quite save.

+-------+-------+-------+-------+-------+-------+-------+------+

SIO accepts any IRQ from 1 to 15, and any COM from 1 to 255

+-------+-------+-------+-------+-------+-------+-------+------+

To unload SIO type: SIO /U

+-------+-------+-------+-------+-------+-------+-------+------+

Check your hardware computers manual for 'slot' pin and IRQ con-

nections to find out where any IRQ is located and be sure to re-

wire correctly when changing any IRQ number !

+-------+-------+-------+-------+-------+-------+-------+------+

Final note, when you start using SIO  for the first time and you

know  you have done everthing  correctly, but  it still  doesn't

work, reset the computer completely and try again.

& - SIO  a Serial Hard/SoftWare IO Convertor - Version 1.1  (C) Saesoft  1990 -

History:

+-------+-------+-------+-------+-------+-------+-------+------+

v1.1 - Fixed 'divide error' in previous 1.0 version.