NetMon for NetWare, version 2

NETMON stands for NETwork MONitor. NETMON2.NLM is a NetWare loadable module, which - loaded on a given server - is capable to interactively monitor network traffic on one or more selected network interface cards.

Features at a glance

NETMON main screen provides quick overview of what is happening on the network segment, monitored NIC is connected to. Semi-graphical, colored histograms as well as various counters (TX, RX, packets, bytes, peak,...) display information about selected bound NIC in comprehensive format. In case there are several NICs bound simultaneously, green and yellow lights in folder corner can provide information about current network activity on “hidden” boards.

Capture screen allows packet capturing on a selected NIC.    Because switching to and from promiscuous mode is also supported by NETMON for NetWare, not only the traffic which belongs to the server can be seen/captured, but in fact any packet on the segment to which the server is connected. Capture screen provides comprehensive information about captured packets, and can be effectively used for first quick analysis of any server/client communication problem.

Captured data can be filtered "on the fly", according to what the need is. Filtering is based on source/destination MAC address or IP address, on protocol type, or on any given known offset. Once packets are in the buffer, they can be shown and dumped on the screen, or saved into the file for later detailed analysis. File formats supported are compatible with Novell's LANalyzer file format (TR1), and    SNIFFER’s file format (CAP).

NETMON for NetWare supports post-filtering operations over captured packets. Simple yet powerful filter command interpreter has been built into this software.

One of the big advantages of NETMON for NetWare is that there is no need for any extra PC or other special device when capturing the traffic between the server and the client PC, while being on network segment equipped with intelligent switching devices. Such intelligent devices by default “hide” the network traffic which does not belong to the port the server is connected to. It means monitoring traffic between NetWare server and the client PC, or between two NetWare servers, using client based packet sniffers, can be complicated, or even impossible.    NETMON for NetWare significantly helps in such situations.

NETMON for NetWare version 2 recognizes and supports multiple boards/interfaces on the NetWare server concurrently, and also supports switching to/from promiscuous mode, which is needed for capturing the whole traffic on the given network segment.

This version of NETMON for NetWare supports Ethernet 10MBit, 100MBit (Fast), as well as 1000MBit (Gigabit) technology, and can be loaded on NetWare 5.x (with SP4), NetWare 6, and on the latest NetWare 6.5 (Nakoma).

NETMON for NetWare also supports Token-Ring 4MBit, TR 16MBit, and TR 100MBit technologies,    but the support here is somehow limited. FDDI 100Mbit technology is officially not supported, but is supposed to work.

What’s new in NETMON2.NLM ver. 2.x?

	Internal capture engine is completely overwritten and more efficient.

	New user interface with new and differently looking statistics.

	Support for concurrent multiple boards monitoring, up to 4 interfaces.

	MT-safe, multi-processor support (not CLIB version).

	Alternate monitoring/binding method using PRESCAN stack.

	Significantly improved WRAP buffer technology.

	Significantly reduced o.s. and processor overhead.

	Enhanced filtering possibilities.

	Program parameters are stored locally.

	Support for CAP file format.

	Synchronous packet capturing.

Installing NETMON2.NLM

NETMON for NetWare is purposefully written in such a way that it does not require any special settings, configuration files, or other complicated dependence to be loaded and run. The idea is to have “load and go” tool. All you need to start NETMON2.NLM    on your server console is a valid software license, and supported NetWare o.s.

NETMON for NetWare ver. 2 can be loaded on NetWare 4.x, 5.x, and 6.x. Simply copy your valid NETMON2.NLM software license to SYS:SYSTEM directory (or to any other directory on your NetWare server and the installation is done !

NETMON2.NLM versus LibC / CLib

NETMON for NetWare ver.2 has been originally written for LibC. It means necessity to load NETMON ver.2 on servers equipped with this latest Novell library technology - namely    NW 5.1SP4 (and higher), or    NW 6.x. As there is continuing demand from users who are still using older CLib technology (NW 4.x, iChain, etc.), we have decided to back-port NETMON for NetWare ver.2 to this legacy CLib environment. Now there are two flavours of NETMON for NetWare ver.2 - one for LibC, and the other one for CLib.    From the funtionality point of view    the only difference is    that Clib version does not support multiple processor platform, and all threads are executed on processor 0 only.

Starting NETMON2.NLM for the first time

Type “NETMON2” on your server console, and NETMON for NetWare loads. For the very first start you will get an empty NETMON interface, without any bound information, with all counters reset to zero, and with all internal variables set to “default”.

Storing NETMON settings and starting the application for the next time

Once NETMON is up and running, configured to collect the information from various interfaces, and showing it in various formats, all application related settings are stored internally on the server, and are automatically used during the next start.

The only exception is the capture process itself, which does not start automatically when NETMON is loaded. The reason for that is to avoid possible timing issues and memory allocation issues.

In case there is a need to avoid NETMON for NetWare starting with previously stored values, one of the following parameters can be used:

RESET	- means: delete all previously stored values during the start, and reset them to default

IGNORE	- means: ignore values which are currently stored, start with default, and do not store when exiting

Example:	LOAD NETMON2 IGNORE

Binding NETMON to an existing interface

To monitor NIC and provide any information, NETMON must be first bound to any existing NIC. NETMON for NetWare version 2 offers two different bind methods (or technologies): ODI Monitor (also used by previous NETMON version 1.x), and a new method: PRESCAN stack. ODI monitor is a default method, which is always automatically selected when user hits <F2> function key. In case ODI monitor cannot be used for whichever reason, NETMON automatically tries in the next step to bind using PRESCAN stack. Both RX and TX channels are automatically bound (if possible) when using this default bind option.

An alternative bind process is invoked when pressing <Ctrl><B> combination. In this case user can select, which method should be used for binding - whether ODI Monitor, or PRESCAN stack. Also only one channel can be bound, and the other one can be ignored by pressing <ESC> key.

After being bound to an interface, NETMON for NetWare automatically clears all related counters and starts to gather information.

Unbinding NETMON from an existing interface

Bound interface can be released from NETMON2.NLM by pressing <F3> key. In case capture process for the board is in active state, the UNBIND command is ignored, and warning message appears on the main screen.

Switching between stats windows

Running NETMON for NetWare provides three different stats windows:

1)	Basic statistics (total, peak, current,… for RX,TX)

2)	Distribution statistics (packet size and protocol)

3)	Capture process statistics (provide information about all currently active capture processes)

Toggling among these stats windows can be done using <ALT><F3> combination.

Histogram windows

NETMON for NetWare provides histograms for last minute and last 12hours (24 hours), which can be combined in three different views:

a)	Last minute & Last 12 Hours

b)	Last minute only

c)	Last 24 hours only

By default, histogram’s scale is automatically adjusted - scale is re-computed on the fly, so that the last known max. value for a given period is always displayed as a MAXIMUM. In this case MAXIMUM value is always displayed in histogram’s right bottom corner.

<ALT><F1> key combination for minute histogram, or <ALT><F2> key combination for hour histogram, makes it possible to switch to 1:1, 1:10, or 1:100 scale instead.

Resetting internal NETMON counters

NETMON internal counters for currently selected board can be set to zero by hitting <F7> key.    This does not have any influence on counters for other bound boards.

All internal counters for all bound boards can be set to zero synchronously by pressing <Ctrl><F7> combination.

Starting and stopping capture process synchronously

Capture process for selected bound interface can be started directly from CAPTURE SCREEN. For more information on that please see paragraph…

If you have more interfaces bound to NETMON2.NLM and you need to start or stop capturing synchronously on all of them, you have to do that from main window.

<Ctrl><S>	starts capturing process

<Ctrl><E>	stops capturing process

<Ctrl><R>	stops capturing process, resets related counters to zero, and clears buffers

Starting NETMON with full functionality from AUTOEXEC.NCF

In case there is a need to use NETMON    from AUTOEXEC.NCF for semi-automatic packet capturing, parameter FULL can be used to initialize capture buffers and threads during the NETMON load process. In this case NETMON2 starts with previously stored settings (including promiscuous mode). If it is not possible for some reason to allocate required memory size for capture buffers, default buffer size (1MByte for full version) is used.

Example:	LOAD NETMON2    FULL

Notice: this feature has been added from version 2.01

Locking console screen in NETMON

NETMON2.NLM provides functionality for temporary locking console screen. Locked console does not allow any activity from the keyboard, except for entering unlocking password,

(or entering internal debugger which stops the whole o.s.).

This feature can be useful if you wish to have NETMON stats displayed on console, but you don’t wish at the same time someone else can have access to the server via console. After pressing <Ctrl><L> combination, NETMON2.NLM will ask for unlocking password, and then it will lock the console. Please be careful when entering the password, because NETMON does NOT asks twice!

In case you forget the password or type the password incorrectly, you have only a little chance to unlock the console. It can be done from internal debugger, but you need to have very good o.s. knowledge for that.

NETMON for NetWare: main window functionality overview

Here is the list of options available from the main NETMON.NLM program window:

<TAB>		Switch to the next bound board (if there is any!)

<LeftArrow>	Switch to the next bound board (if there is any!)

<RightArrow>	Switch to the next bound board (if there is any!)

<F1>		Display quick help for main program window

<F2>		Bind (auto) selected logical board to NETMON

<F3>		Unbind already bound logical board from NETMON

<F4>		Show logical board basic information (MLID info)

<F5>		Toggle between promiscuous mode and normal mode

<F6>		Reset all NETMON counters for given board

<F9>		Open (or switch to) capture screen for bound board

<F10>		Quit the NETMON application

<Alt><F1>

<Ctrl><F1>	Toggle scale in LAST MINUTE histogram

<Alt><F2>

<Ctrl><F2>	Toggle scale in LAST HOUR histogram

<Alt><F3>

<Ctrl><F3>	Toggle counters, statistics, and capture window

<Alt><F4>

<Ctrl><F4>	Toggle LAST MINUTE & LAST HOUR histograms

<Ctrl><R>	Reset counters for all bound boards

<Ctrl><A>	About this program

<Ctrl><B>	Alternate bind (manual) for selected logical board

<Ctrl><E>	Sync stop packet capturing on all bound boards

<Ctrl><L>	Lock server console in NETMON screen

<Ctrl><R>	Sync reset packet buffers on all bound boards

<Ctrl><S>	Sync start packet capturing on all bound boards

Opening capture window and initializing capture process

NETMON for NetWare, when bound to a given NIC, is capable to capture packets being passed through that NIC.    For that, new process (thread) must be started by pressing <F9> key from NETMON’s main program window. But only in case NIC is bound to the NETMON, and there is no capture process yet, new capture process is launched. In case capture process has already been initiated (before), hitting <F9> key simply switches view to capture process window (and back).    Only one capture process at the time can be started for one bound NETMON monitor.

Starting and stoping capture process

Once new capture thread has been opened and initialized, capturing can be launched immediately by pressing <F5> key. Another hit to <F5> key stops capturing and displays results.

Packet capturing can run in two different modes - with or without wrapping buffer.

Storing captured data to file

Captured data can be stored to a file on the server. Hitting <F2> key invokes dialog window, where new file name for stored data can be entered. Only DOS name format is supported in this dialog. File name can be either absolute - including volume name and all directories, or relative - filename only.

Similarly, by pressing <F3> key you can invoke dialog window for loading data from existing TR1 or CAP file to internal capture buffer. You need to allocate enough space before otherwise NETMON can refuse to load data.

Setting capture filter and properties

Internal packet buffer size, mode it operates in, and capturing/filtering features can be changed when hitting <F4> key. For more information about packet filter options see the following paragraph.

Please keep in mind that there are some limitations for buffer size/mode changes:

1)	Buffer size cannot be modified in case buffer operates in WRAP mode, and there are currently some data captured.

2)	Switching from WRAP mode to RUN ONCE mode cannot be done in case buffer is not empty.

For backward compatibility with NETMON v.1, and also for easier manipulation, buffer size can also be changed directly by hitting <F7> key. Similarly, buffer mode can be easily switched by hitting <F8> key.

Packet filtering options

NETMON for NetWare supports 'on fly' packet filtering and allows certain capture criteria (filters) to be set for incoming packets. These criteria are based on source/destination MAC address, source/destination IP address, type of protocol, and binary string (up to 8 bytes) found in the packet. The logic behind filtering is based on AND operation, so that the filtering engine requires only a minimum of time for decision, and does not create almost any delay in packet delivery:

{<MAC or IP> matches }    AND { binary string at offset matches}    AND { protocol matches}

The following syntax is supported in the packet filter window:

MAC/IP:	XXXXXXXXXXX		Src/Dest MAC address in HEX format

DDD.DD.DD.DDD	Src/Dest IP address in dotted DEC format

<any>			This means broadcast MAC or IP address

<this> or<me>	Internally replaced by the MAC address of the given bound logical board

Offset/Value:	XXXX			Any number in HEX format. Offset means position

from the packet begin, value is binary pattern with the length between 1-8 bytes.

Packet Size:	DDDD			Any numeric value in DEC format, or '<full>'

Buffer Size:	DDDD			Any numeric value in DEC format, incl. 'KB', or 'MB'

Buffer Wrap:	YES or NO

File Name:	Relative or absolute path in DOS name format with 'TR1' or 'CAP' extension.

Protocols:				Can be selected/toggled by pressing SPACE key

Important notice:

Current NETMON2.NLM does NOT have implemented filtering for TX channel in PRESCAN stack. The reason is that at the time PRESCAN stack is invoked by call-back routine, transmitting packet has not been assembled by o.s. yet. That’s why already implemented filtering routines, which work smoothly for ODI monitor and RX PRESCAN stack, cannot be used here.

Resetting capture buffer

Capture buffer can be reset by pressing <F7> key. All data stored in the buffer will belost

Post-fitering packets in capture buffer

NETMON for NetWare supports post-filtering operations over captured packets.    Simple yet powerful filter interpreter has been built into NETMON. Below are the syntax and supported keywords and expressions allowed in this NETMON version:

<+|-><keyword> = <expression | data

+|-    FRAME	= 	ETHER_II | ETHER_8022 | ETHER_8023 | ETHER_SNAP

+|-    FRAME	= 	TOKEN_SNAP | TOKEN_8022

+|-    PROT	= 	IP | IPX | ARP | RARP | ATALK | ATARP | NBIOS

+|-    PROT	= 	ICMP | UDP | TCP | FTP | TELNET | SNMP | LDAP

+|-    PROT	= 	RIP | DHCP | SMTP | SLP | HTTP

+|-    PROT	= 	SPX | NWRIP | SAP | NLSP | NCP | NDS

+|-    MAC	= 	hhhhhhhhhhhh        (hexadecimal format)

+|-    IP	= 	ddd.ddd.ddd.ddd        (decimal format)

+|-    OFFSET      = 	hhhh, DATA=hhhhhhh...      (hexadecimal format)

Only one expression per command is allowed, space characters are ignored, lower cases are always translated into upper cases.

Examples:

+FRAME=ETHER_II	= only packets in EthII envelope remain

+MAC=00506E298744	= only packets with MAC specified (either source or destination)

-PROT=SPX		= filter out all SPX packets

-OFFSET=4A, DATA=39A0442B06A1

NETMON for NetWare: capture window functionality overview

Here is the list of options available from NETMON2.NLM capture window:

<F1>		Display quick help for capture window

<F2>		Store packets from buffer in a trace file (CAP or TR1 format)

<F3>		Load packets from a trace file (CAP or TR1) to buffer

<F4>		Show and/or set capture filter (pre-filtering)

<F5>		Start/stop capture process

<F6>		Reset capture buffer and capture counters

<F7>		Change buffer size for captured packets

<F8>		Toggle between RUN ONCE and WRAP buffer mode

<F9>		Switch back to main NETMON2.NLM screen

<F10>		Close capturing window and return back to main NETMON2.NLM screen

<ENTER>	Open Packet Decode window

<INSERT>	Select/deselect highlighted packet

<Alt><C>	Toggle between colored/grayed decode output

<Alt><D>	Deselect all (previously) selected packets

<Alt><F>	Post-filter from buffer based on given criteria

<Alt><G>	Go to the packet number XXX

<Alt><P>	Store packet trace summary into a text file

<Alt><R>	Reset (post)filtered packets

<Alt><S>	Search for a specific (char) string from current packet position to the end of capture buffer

Following options are available in the Packet Decode window:

<Ctrl><PgUp>	Show previous packet (one packet up)

<Ctrl><PgDn>	Show next packet (one packet down)

Support for NoRM (NetWare Remote Manager)

NETMON2 also support NoRM interface from version 2.10. It means you can use most of NETMON2 features easily by pointing anc clicking through HTML interface. Link to NETMON2 can be found in NoRM under APPLICATION MANAGEMENT.

You can alternatively use the direct link to NETMON2 tool:

Unsecured access: http://<server_ip_address>:8008/netmon2

Secured access:          https://<server_ip_address>:8009/netmon2

By default NETMON2 is loaded on server in secured state and requires secured access. It also requires that remote NoRM user must be logged to NoRM interface, and must have both Admin and console operator privileges in order to use this tool. If you need unsecured/unprivileged access through NoRM, you can load NETMON2 with parameter NOSECURE:

LOAD    NETMON2    NOSECURE

Packet buffer limits

Basically there is no limitation in packet buffer size, or in number of packets, which can be handled by NETMON2. You can allocate as much memory for packet buffer(s) as operating system NetWare allows.

There may be limitation when using NoRM HTML interface, and this limitation comes from HTML browser memory requirements. We have succesfully tested 500.000+ packets displayed and handled in one single HTML window. However in such extreme case PC client memory requirements are huge, you would need 2 or more GBytes of memory on the client station.

Known limitations or issues

	Current NETMON2.NLM does NOT have filtering for TX channel in PRESCAN stack implemented. The reason is that at the time PRESCAN stack is invoked by call-back routine, transmitting packet has not been assembled by o.s. yet. That’s why implemented filtering routines, which work smoothly for ODI monitor and RX PRESCAN stack, cannot be used here. That's also why captured frame header from TX PRESCAN assembled by NETMON2.NLM may not be 100% identical with the frame header created later by o.s. and sent to the wire.

	NETMON.NLM does not provide full and detailed packet analysis.    This is not the main purpose of this tool. If you are in such a need then please export trace in CAP or TR1 format and use any appropriate client based packet analyzer.

	Time for packet reception does not support possible daylight saving time setting on the given server.

	Utilization shown in NETMON2 may go above 100%. The reason is that NIC may be running in duplex mode, and there is no way for NETMON2 to figure out if NIC is in duplex mode or not. Historically, NetWare o.s. does not provide any unified interface for this specific information. That's why NETMON2 can show utilization beyond 100%.    Part of utilization is calculated from transmit channel numbers (TX), the other part from the receive channel (RX) numbers.    Theoretically there is a possibility to get up to 200% utilization, or even more if NIC is running in special buffered mode.    All calculations are done using system timer, and the information coming from NIC. It may happen that the timer fails to provide exact 1000ms interval, and at the same moment both RX & TX buffers may be "overloaded" with data. This would for sure result in incorrect calculation beyond theoretical maximum of 200% of utilization.

	Multiple concurrent user access from NoRM is    not handled with this version of NETMON2 (2.10). It means two (or even more) remote users from remote place can concurrently manage one single NETMON2 instance through HTML.

Limitations of the date-stamped version

	It will work only until certain date is reached (please check output on system console when loading NETMON2, or see info in ABOUT window).

	Capture filter cannot be set or modified.

	Memory adjustment for capture buffer cannot be done, and memory for captured packets is significantly limited (8 KBytes only).

List of changes / fixes

April    2004	Version 2.10, build 1027

Support for NoRM implemented (NetWare Remote Manager)

Sept    2003	Version 2.03, build 1015

Fixed issue with Tx Monitor filtering.

July    2003	Version 2.02, build 1013

Back-ported to CLib. Now there are two flavours available - one for LibC, and one for CLib.

July    2003	Version 2.01, build 1011

Added functionality which allows NETMON2 to be started with full capturing functionality during the load. 		This feature is switched with "FULL" parameter, and can be used for example for launching NETMON2 			from AUTOEXEC.NCF.

June 2003      	Version 2.00a, build 1009

Fixed the cosmetic problem with information format on CAPTURE INFO screen

June 2003      	Version 2.00a, build 1008

Modified code for VIRTUAL IPX interface handling.

Modified TR1 file output to be readable with ETHEREAL software (there is a small bug in ETHEREAL).

June 2003	Version 2.00a, build 1007

Fixed bug which caused abend on SCMD or VIRTUAL IPX interface.

June 2003	Version 2.00, build 1006, first official release

Final notice

Many existing network utilities use the name “NETMON”. They are either written for WIN32, or for Linux/Unix. This NETMON for NetWare (NETMON2.NLM) has nothing in common with them, except for the name and the very basic idea, which is network monitoring.

NETMON for NetWare functionalities, features, and interface are exclusively based on original code written from the scratch and owned by Rostislav Letos, ROLETOSoft, Czech Republic.

Copyright

Copyright 1999-2004,    Rostislav Letos, ROLETOSoft

ROLETOSoft is registered mark.

April 27, 2004,    document version: 1.1.0