ARCDIAG

ARCNET DIAGNOSTIC PROGRAM

USER GUIDE

Version 2.2

For use with

Novell Netware/SM/RX Type Networks

(c) Copyright 1986, 1989

by

Olympic Software Co

2601 S. State Street

Ann Arbor, MI  48104

(313) 994-3666

All Rights Reserved

Worldwide

INTRODUCTION

The ARCDIAG program is designed to be a service tool for network

administrator personnel for a network that uses ARCNET type

hardware (see specifications section for compliance).   The ARCDIAG

program includes two diagnostic sections that will help during

installation of an ARCNET network.  The diagnostic software can be

operated at any workstation during installation of the network or

at any time while running the Novell Network Operating System.

NOTE: The ARCDIAG program is not intended to

isolate Novell Software problems except in

those cases where the physical network

hardware is the prime cause of the problem.

The program exercises the network card by testing all of its

internal functions, RAM, RIM etc., and then displays a map of

active nodes and packet counts.  The latter also provides an

indication of any reconfiguration that might be occurring.

THE DISTRIBUTION SET

The files contained in the distribution set are:

ARCDIAG.EXE	The executable ARCDIAG file

ARCDIAG.DOC	This documentation file

ARCDIAG.WP5	Documentation file in Wordperfect 5.0 format

USAGE/COPYING LICENSE

The software described herein and the accompanying documentation

is copyrighted and the sole property of Olympic Software Co.

The information in this document is subject to change without

notice and should not be construed as a commitment by Olympic

Software.  Olympic Software assumes no responsibility for any

errors that may appear in this document.

The software described in this document is furnished under license

and may only be used or copied in accordance with the terms of such

license.

WHAT IS SHAREWARE

Computer users and programmers have been exchanging software since

the dawn of computers. Only recently, however, has the concept

blossomed into an effort to provide low-cost alternatives to

conventional commercially marketed software.  Known as Shareware,

this concept provides a source of high quality software products

to users, and permits them the opportunity to "try before you buy".

Also, because expensive advertising and distribution costs are

avoided, the cost of Shareware tends to be significantly lower than

for a commercial product of equivalent quality and functionality.

Shareware authors hold copyrights on their programs.  Just as for

commercial software, the Shareware author's software is protected

by U.S. and international copyright law.

Shareware, unlike its commercial counterpart, may be freely copied

and passed to others.  Like commercial software, Shareware is a

"for-sale" product and Shareware authors request or require that

people who try their software and decide to use it send a

"registration fee" (approximately equivalent to purchasing the

software).  In addition to granting a license to use the product

for this registration fee, authors offer a wide variety of services

to include (depending on the program) printed documentation, disks,

advanced versions, and/or telephone support.  Some companies also

offer free updates and commissions.  But, the most important

benefit of registration is that you receive a legal license to

continue using the software.

Some Shareware authors have formed the Association of Shareware

Professionals (ASP) dedicated to establishing recognized standards

for Shareware programs and authors.  The authors share a code of

ethics and have agreed to adhere to the ASP's standards for

software and documentation.  The ASP is also striving to help disk

dealers, the media, and the public to understand the nature and

benefits of Shareware.  Through cooperation and mutual standards,

the ASP hopes to promote Shareware as a viable alternative to

commercial software.  Not all authors belong to ASP, but as more

join, Shareware standards will be more universally followed.

Your Rights and Obligations

License 	A limited license is granted to freely use, copy and

distribute the product subject to the following

restrictions:

YOU MAY NOT

Sell and/or distribute the product for a fee, either

alone or along with other products, without express

written permission of the copyright holder.

Grant sublicense, leases, or other rights in the software

to others.

Modify or remove the Copyright notice from the program,

diskette or the documentation.

Modify the program, diskette or documentation and/or

merge it into another program or product for any machine.

Distribution

ARCDIAG is specifically authorized for distribution,

under the SHAREWARE principle, via public electronic

bulletin boards or by individual copying. Any

distribution thus made must be in the form of this entire

.ARC file, UNALTERED, including the program executable

files, data files, and this or any other accompanying

documentation files. NO OTHER FORM OF DISTRIBUTION,

INCLUDING DISTRIBUTION AS PART OF A LARGER PACKAGE, NOR

DISTRIBUTION FOR A FEE (other than a nominal copying

charge not to exceed $10.00), IS AUTHORIZED WITHOUT

SPECIFIC WRITTEN PERMISSION OF THE COPYRIGHT HOLDER.

Limited Warranty

THE PROGRAM IS PROVIDED "AS IS" WITHOUT WARRANTY OF ANY

KIND, EITHER EXPRESSED OR IMPLIED, INCLUDING BUT NOT

LIMITED TO THE IMPLIED WARRANTIES OF MERCHANTABILITY AND

FITNESS FOR A PARTICULAR PURPOSE. THE ENTIRE RISK AS TO

QUALITY AND PERFORMANCE OF THE PROGRAM IS WITH YOU.

SHOULD THE PROGRAM PROVE DEFECTIVE, YOU ASSUME THE ENTIRE

COST OF ALL NECESSARY SERVICING, REPAIR OR CORRECTION.

SOME STATES DO NOT ALLOW THE EXCLUSION OF IMPLIED

WARRANTIES SO THE ABOVE EXCLUSION MAY NOT APPLY TO YOU.

THIS WARRANTY GIVES YOU SPECIFIC LEGAL RIGHTS AND YOU MAY

ALSO HAVE OTHER RIGHTS WHICH VARY FROM STATE TO STATE.

Limitation of Remedies

IN NO EVENT WILL OLYMPIC SOFTWARE CO. BE LIABLE FOR ANY

DAMAGES INCLUDING ANY LOST PROFITS, LOST SAVINGS OR OTHER

INCIDENTAL OR CONSEQUENTIAL DAMAGES ARISING OUT OF THE

USE OR INABILITY TO USE SUCH PROGRAM EVEN IF OLYMPIC

SOFTWARE HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH

DAMAGES OR ANY CLAIM BY ANY OTHER PARTY.

Software/Shareware Dealers and Distributors

Please write to the Author for permission to include this

product in your collection PRIOR TO offering it for sale

or distribution. Inquiries are encouraged, and will be

responded to as quickly as possible.

START UP

The program can be run from the DOS prompt by typing:

ARCDIAG  [-mxxxx] [-ixxxx] [-ffile descriptor] [-ttype] [-b]

Start Up Options:

-b	      Option for removing the preliminary board

test.

-ffile 	      Option for specifying the location of the

logical name file - ARCDIAG.DAT.  40

characters maximum. (Unlimited version

only)

-ixxxx	      Option for operating ARCDIAG at an I/O

address of xxxx (hex). ARCDIAG defaults

to 02E0 (hex).  Must use all 4 digits.

(Unlimited version only).

-mxxxx	      Option for operating ARCDIAG at a RAM

memory location of xxxx (hex).  ARCDIAG

defaults to D000 (hex).  Must use all 4

digits.  (Unlimited version only).

-q	      Option allows board test only operation.

Place [ARCDIAG -q] in the AUTOEXEC.BAT

file for automatic test of ARCNET card on

power up.  If an error condition is

detected, the nature of the error is

displayed and the DOS error level is set

to 1.  Otherwise, nothing is displayed,

and the DOS error level is kept at 0.

(Unlimited version only).

-ttype	      Option for selecting different network

interface cards.  ARCDIAG defaults to

standard SMC type cards with memory mapped

RAM and I/O mapped RIM controllers.

(Unlimited version only).

ArcDiag can be loaded and run in any file server or work

station, providing there is a floppy drive available.

The ARCDIAG.EXE file may be copied onto and executed from the

network hard disk, but keep in mind that board testing is

disabled if the Novell Shell is loaded.

Type ARCDIAG at the DOS prompt of the drive containing

ARCDIAG, whereupon, if the Novell Shell is not loaded, the

board test screen shown in Figure 1 will appear:

Network Board Test

PASSED: RAM test

No Network Boot PROM found

PASSED: Internal RIM test

PASSED: Physical node address is: 1

PASSED: ETS test

PASSED: POR test

Figure 1 - Board Test Screen

BOARD TESTS

Unless option [-b] is included in the startup command, the

board test function automatically begins when entering

[ARCDIAG] at the DOS prompt.

Board Types

ARCDIAG has been set to function on most Arcnet

boards, with the exception of Nestar.

By including [-ttype] in the command line as a start up

option, different board types may be selected, e.g. [-

t1], [-t2], etc.  Following is an example of the command

line:

ARCDIAG [-mxxxx] [-ffile descriptor] [-ttype]

[-t1] is the default and is used for most Arcnet cards

that use memory mapped RAM and I/O mapped RIM controllers

such as SMC, Novell, Earth, etc.

[-t2] is used for memory mapped RIM controllers, such as

Thomas Conrad, Arcmaster, etc.  When the [-t2] option is

used, the [-I] option is disabled.  (Unlimited version

only).

Note:  To select an Arcmaster card

that has RAM at E000 hex, enter the

following command line: (Unlimited

version only).

ARCDIAG -me000 -t2

The following tests are made during the board test phase:

NOTE:  When ARCDIAG is first executed, board

testing is disabled if the user has already

loaded the Novell Shell.

RAM Test.--ARCDIAG exercises the ARCNET board's RIM RAM.  All

2K of RAM are tested for shorted and open bits.  Also, the

address lines are tested.

An error message is displayed if the RAM fails the test or if

the DIP switch setting of the memory address set is incorrect.

A PASSED message is displayed if operation is verified.

Network Boot Prom Test.--This test indicates whether or not

a network boot prom is installed.

RIM Test.--The token controller IC (9026) is tested in the RIM

test.  The network microprocessor (9026) can perform its own

internal diagnostics.  When the test is run, approximately 95%

of the processor's operations are exercised.  If this test

fails, replacement of the network card is recommended if it

is verified that the I/O address DIP switches are set

correctly.

A PASSED message is displayed if operation is verified.

SID Test.--During the SID test, the Controller source ID is

located and read.  The returned value from this test is the

station node address.  The test will fail if it is at zero.

If failed, make sure the address switches are set to an

address greater than zero.

A PASSED message is displayed if operation is verified.

ETS Test.--The ETS test verifies the proper location of the

(E)xtended (T)imeout (S)etting jumper.  Newer ARCNET cards may

not have a jumper.  Therefore, the card needs to be replaced

if an error occurs.

A PASSED message is displayed if operation is verified.

POR Test.--During this test, a power-up cycle is activated and

verified.  The 9026 RIM processor is reset and the ARCNET card

is placed into an operational state.

After performing all of the above tests, the program will halt for

approximately three seconds, then cycle into the Network Tests.

The Board Tests may be continued by selecting [Board] from the

Lotus-type menu at the top of screen, or by merely pressing [B].

NETWORK TESTS

Network tests function both in and out of the Netware shell.

However, the file server and workstations can be tested only if

they are powered on.

The Network Test Grid shown in Figure 2 appears after ArcDiag has

been started and the Board Tests are completed.

Map      Count     Names    Format     Board     Print     Reset     Quit

������������������������������������������������������������������Ŀ���������Ŀ

� Dec   1   2   3   4   5   6   7   8   9  10  11  12  13  14  15  ��         �

�   �������������������������������������������������������������Ŀ��         �

�  0�  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �  �����������ĳ

� 15�  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �  ��� Rcn/Min �

� 30�  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �  ���         �

� 45�  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �  �����������ĳ

� 60�  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �  ��� Station �

� 75�  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �  ���         �

� 90�  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �  �����������ĳ

�105�  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �  ��� Address �

�120�  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �  ���         �

�135�  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �  �����������ĳ

�150�  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �  ��� Packets �

�165�  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �  ���         �

�180�  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �  �����������ĳ

�195�  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �  ��� Recnfgs �

�210�  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �  ���         �

�225�  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �  �����������ĳ

�240�  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �  ��� ActNode �

�   ��������������������������������������������������������������ٳ�         �

�������������������������������������������������������������������������������

ARCDIAG V2.2  Limited    Copyright Olympic Software  1989

Figure 2 - Network Test Grid

Each dot on the grid represents a node.  As noted by the cursor

rapidly moving across the grid, the program polls all 255 nodes.

When a file server or workstation is powered on, the node number

or node name will be displayed in the place of the dot.

Information Windows  Information from the network diagnostics

is provided in the windows on the right side of the screen.

The first window contains current date and time.  Following

is a description of the information contained in the other

windows.

Rcn/Min  Indicates the number of reconfiguration per

minute, relative to (1) the number of stations being

powered on or off of the network, and (2) any type of

fault condition occurring on the network.

This information may be used as a gross indication of

network fault conditions (see "Recnfgs" below).

Station  If the Novell shell is running, the station

number on which ARCDIAG is running is displayed in this

window.  This number may be referenced to the station

number which appears on the file server monitor.  If the

shell is not loaded, the message "No Shell" is displayed.

Address  The physical address, or the dip switch setting,

on the network interface card.

Packets  The number of packets that have occurred on the

network since the last reset.  This information indicates

relative network activity.

Recnfgs  Reconfiguration bursts are counted

accumulatively and displayed in this window.

Reconfiguration bursts increment whenever any of the

following occur:

1.	a workstation is powered onto the network.

2.	a workstation is powered off of the network

(intermittent).

3.	a network card is faulty.

4.	the network connecting cable is faulty.

A rapidly increasing reconfiguration count indicates

a faulty network (refer to "Network Troubleshooting,

Network Tests").

ActNode  The number of nodes that have powered onto the

network.

FUNCTION SELECTION

At the top of the Network Test screen is a "Lotus" type menu

for selecting various functions, such as "Map," "Count,"

"Names," etc.  To select a particular function, either move

the cursor key to highlight the function and press [Enter],

or merely type the first letter of the function label, e.g.

[M] for the "Map" function.

Note that a "help" statement, corresponding to each function,

appears at the bottom of the screen whenever a function is

highlighted.

"Map" Function  Selection of this function displays all nodes

on a grid, each node being indicated by a dot.  The number

range is from 1 to 255.  The display is arranged in a matrix

form.  Nodes are displayed 15 at a time along the horizontal.

Therefore, nodes 1 through 15 are displayed on the first line,

nodes 16 through 29 on the next line, etc.

Stations that are powered on are indicated by a node number

or node name, in the place of a dot.

"Count" Function  In this mode, packets from each station are

monitored and displayed on the grid.  Packet counts can be

viewed as an indication of activity from each node.  Those

nodes that transfer packets most frequently will increment the

packet count much more rapidly.

Although the program is not fast enough to catch all packets,

relative packet increases are displayed.  Therefore, the nodes

that are assigned as fileservers should have packet counts in-

creasing at a faster rate than workstations.

"Names" Function  Any of the 255 nodes can be given an alpha-

numeric designation which is useful for quick recognition of

node locations.  (Unlimited version only).

These logical names can be assigned to physical node numbers

by using the "Names" function.  For example, a file server

that might be located on node 1 may be given the logical

designation "FS1".

When selecting the "Names" function, the following message

will appear at the bottom of the screen:

Enter node number (1 to 255):

At the blinking cursor, enter the number of the node to which

you want to give a logical name, then press [Enter].  The

following message will then appear:

Enter name (3 char max):

At the blinking cursor, enter a maximum of three characters

and press [Enter].  You will then be asked:

Save (y/n) <return = yes>

If your entry is correct and you want to save it to the

ARCDIAG.DAT file on the disk, press [Enter], since "Enter" or

"Return" is the default.  Otherwise, press [N] to ignore the

entry.

The program uses the file ARCDIAG.DAT to store logical names

for physical node numbers.

The ARCDIAG.DAT file can be located in either the default

drive or when connected to a Novell network it can be located

in the Z: (public) directory.  Optionally a command line

argument (-f) can be used to tell ARCDIAG where to locate and

what name to use for the logical name file (see "Start Up

Options").

"Format" Function  Physical node numbers can be displayed on

the node map as either decimal or hexidecimal numbers.  The

current mode of operation is displayed in the upper left hand

corner of the display.

Pressing the [Enter] key more than once will toggle the

display mode back and forth between decimal and hexidecimal.

"Board" Function  This function runs diagnostics on network

interface cards.  In order for the board diagnostic test to

operate, however, ARCDIAG cannot be running in conjunction

with the Novell operating system.

When this test is initiated from the menu at the top of the

screen, it will continue until the space bar is pressed or

until an error is found.

"Print" Function  If an IBM graphics compatible printer is

available, use PrtSc to produce an exact copy of the map

including graphic borders.  (Unlimited version only).

If an IBM graphics compatible printer is not available, or if

you want to label and date the node map, select the "Print"

function on the menu, whereupon the following statement will

appear at the bottom of the screen:

Enter map name:

Type a name for the map, then press [Enter].  A node map with

time and date stamp will be sent to the printer.  All borders

are converted to characters that can be printed on any type

of printer.

The printing program does not send a form feed character at

the end of printing allowing two maps to be printed on one 8

1/2 by 11 inch piece of paper.

"Reset" Function  Select this function to reset all counters

to zero and reformat the grid.

"Exit" Function  Select this function to exit the program.

SPECIFICATIONS

RIM RAM Base Address	D000 (hex) segment default

configurable at runtime (Unlimited

version only).

Board I/O Address       	02E0(hex) default

configurable at runtime (Unlimited

version only)

Active Nodes Displayed  	1 thru 255

RIM RAM size		2K bytes

Computer Compatibility	IBM PC/XT/AT or compatible Network

Operating Systems Novell Netware 4.61

and above.  Novell Advanced Netware

1.02 and above.

Network Interface Cards	SMC 8 bit and Novell RX 8 bit

NETWORK TROUBLESHOOTING

Board Tests

Symptom				Corrective Action

RAM Test Fails		      Check network card memory address DIP

switches for proper settings.

RAM defective, replace card.

RIM Test Fails		      Check network card I/O address DIP

switches for proper settings.

9026 RIM controller defective, replace

card

SID Test		      The SID address should display the node

address.  Note:  Do not allow two network

cards to have the same physical address.

ETS Test Fails		      Newer network card does not have a jumper

for extended timeout.  Therefore, if this

test fails, the board needs to be

replaced.

POR Test Fails		      Before, continuing, remove the network

cable and try again.  If the test still

fails, the network card needs to be

replaced.  Disconnecting the cable removes

any type of external interference with the

test.

Network Tests

Symptom			      Likely Cause

Reconfiguration

counter increments

continuously		      (1) no other nodes are connected to the

local network card.

(2) the cable network is defective from

the local card to the rest of the network.

(3) an active or passive hub port is

defective.

Multiple "phantom" nodes

are displayed		      (1) two nodes have been set to the same

address.

(2) a cable connection is not making solid

contact.

(3) the memory address DIP switches are

set incorrectly.

(4) -i or -m options have been set

incorrectly.

When run with Novell

shell - local node is

not displayed		      Nothing is wrong with the display.  The

Novell shell has not passed the proper

physical node address to ARCDIAG.  Works

correctly when not running with the shell.

Fixed in Novell Advanced Netware 2.0 and

above.

ARCDIAG shows OK, but

cannot "find fileserver"

when shell is run	      Make sure the interrupt jumper JP5

is used for all network cards used

with Advanced Netware 1.02 and above.

HOW TO REGISTER ARCDIAG

If you find the program useful, and continue to use it past a

30-day trial period, you are required to register it. You can

register by completing the accompanying registration form and

sending it, with a check or money order in the amount of $45.00 (if

foreign funds, at current exchange rate) to Olympic Software Co.

at the address shown on the registration form below.

Registration provides the following benefits:

Elimination of the initial registration reminder screen.

Enabling of the following features:

Named nodes

Map printing

Command Line arguments

Full Telephone support.

Free major updates for a one year period, sharply discounted

rates thereafter.

Inclusion on our mailing list so that you may receive new

product announcements and notification of new Olympic Software

products.

You will be in compliance with federal copyright law.

USER SUPPORT

Registered users are provided with automatic notification of major

updates of ARCDIAG.  Call (313) 429-7139, a call-back will normally

occur no later than the next business day. Bug fixes or

work-arounds will be mailed to the complainant, if a registered

user, and will also be uploaded to CIS (IBMSW).   Non-registered

users are welcome to call, and will be accommodated on a "time

available" basis. Contact may, of course, always be made through

CIS EMAIL (73227, 3326).

OTHER SHAREWARE PRODUCTS

You may wish to try these other products of Olympic Software:

ARCDIAG.C	Source Code for ARCDIAG		$150.00

ARCDIAG Registration & Comment Form

Name:___________________________________________________________

Company:________________________________________________________

Address:________________________________________________________

City:__________________________ State:_____  ZIP _______________

Telephone:(_____) _____-_______     Date:_______________________

ARCDIAG Version:_____._____

Network Interface Card Types ___________________________________

Comments:

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_________________________________________________________________

_______________________________________________________________

Payment:

Quantity           Cost Each

________           ___$45.00___                   sub-total ____.__

(MI residents only) MI 4% Sales Tax ____.__

TOTAL ____.__

Mail To:       Olympic Software Co

8424 Warner Rd

Saline, MI  48176

#