TESTNET.EXE

version 1.0

By SCOTT TAYLOR

228 Gold Kettle Drive

Gaithersburg,Md 20878

TESTNET this utility is intended to test the performance of Local

Area Networks. It may also be used to test stand alone PC's.

The valuable results of this test will show the total elapsed

time in seconds and total throughput in KB/sec transferred to

disk. Using a SEQUENTIAL file access method tests server disk and

caching performance. Using OVERLAID file access tests reflects

more of the network interface performance as well as local

(workstation) cache techniques. Larger record sizes are handled

more efficiently by most operating systems (especially DOS).

Using larger record sizes (4K and up) demonstrates overall

throughput. Smaller record sizes demonstrates the operating

systems ability to deal with small record blocks. A good example

would be to run this test from a workstation local C: drive. Then

run it again from that workstation but to a network drive.

Usually you will see a significant increase in performance. Also

try running the test with the VERIFY option on/off.

Here is another good example, especially for comparing different

server hardware (disk, memory, cpu). This is especially useful on

Novell networks since the server console displays performance

characteristics. To compare hard disks generate a file size,

using Testnet, that is larger than the total memory installed in

the server. Then you can compare throughput times displaied by

Testnet. This way the file size is to large to fit in cache

memory and tests raw disk I/O.

To compare CPU's (as servers) or cache memory generate a file

size, using Testnet, that is approximately 512K smaller than the

total memory installed in the server. Then set Testnet to read

the file, this will test the raw CPU & Cache memory performance.

Then you can compare the throughput displayed by Testnet. As the

test is in progress you can monitor the server console for peak

"I/O PENDING" and peak "% UTILIZATION".

MENU: The menu keys are UP and DOWN arrow keys to highlight a

menu option. Press RETURN to select a highlighted option, then

input the desired value for that option. Instructions accompany

each menu option when the RETURN key is pressed. <ESC> will exit

TESTNET to DOS.

From the main menu <F1> starts the test. <F2> sets the station

into the MULTI-STATION test mode. Press <F1> when the last

station is set into the MULTI-STATION test mode, then all

stations will begin testing. Each station will then show its

average throughput to/from the file server. All stations running

in the MULTI-STATION mode must be able to access the same root

directory.

Any questions, comments, or suggestions are welcomed !